Lesson plan

Topic 2 MAC and IP addresses

Objectives: Understand what is meant by and the purpose of media access control (MAC) address Understand the structure of a MAC address Understand that there are different types of IP address Know the characteristics and differences between IPv4 and IPv6 Understand that IP address allocation may be static or dynamic

Content

Starter PowerPoint Guide: Topic 2 MAC and IP addresses The purpose of the starter is to show that there is more than one way to address things and that these have different purposes. This establishes the ideas that devices may have both IP and MAC addresses as different ways of showing the address and location of the same device. Main MAC addresses Make sure students are aware that MAC addresses are unique addresses that are given to devices at the point of manufacture. Some networks will try to block unwanted devices by restricting the MAC addresses that can access them. Many devices will have the MAC address stored in firmware, so it is possible for a hacker to change it. As such, MAC addresses are not that useful in network security. The address pool for MAC addresses is large at 281 trillion. This is enough for around 30000 devices for every person on the planet. Whilst this is far in excess of what is needed today, explain to students that every device that connects to the Wi-Fi network, plus all the intermediate routers, plus devices such as Bluetooth headphones all have a MAC address. Hence, the address range needed to be fairly large. Make students aware that MAC addresses are represented in hexadecimal and the first six digits of hex are the manufacturer code. Both these points are given in the specification. Hand out Worksheet 2 and ask students to complete Task 1, Task 2 and Task 3. Topic 2 Worksheet 2 Internet Protocol issues The problem with IPv4 addresses is that there aren’t enough of them. This problem was recognised a long time ago with IPv6 being released as a draft in 1998. IPv4 and IPv6 are not interoperable, and it isn’t possible to simply turn off the Internet and all connected devices and perform an upgrade. As such, the transition to IPv6 is still in progress. In most of the world, there are currently no new IPv4 addresses available, however, existing ones may be sold and transferred. It is also worth noting that multiple websites may share an IP address – this is resolved by looking at the domain name that is part of the web request. IPv6 There are perhaps as many as 100 billion trillion grains of sand in the world, so IPv6 could easily address each of them if they had a NIC. Make sure that students are aware that they need to know some of the fundamental differences between IPv4 and IPv6. Dynamic IP addresses Most home networks that students use will use a dynamic IP address. In the old days of dial-up modems, customers may only be connected to the Internet for an hour or so. Sharing an IP address between users made sense and led to cost savings for the ISP. This makes less sense now, as most users home networks are constantly connected to the Internet. However, typical home ISPs will still give a dynamic IP address that changes. A static address has some advantages, so is normal an additional cost or part of a higher quality ISP. Ask students to complete Task 3, Task 4 and Task 5 on Worksheet 2. Plenary Lots of new terms have been covered in this topic. Ask students to recall them in pairs, then go through the answers that follow. Hand out Homework 2. Topic 2 Homework 2 Topic 2 Homework 2 Answers