Answers

1.	A farmer has a large greenhouse where they grow tomatoes.

There are a number of sensors and actuators that are used to control the heat, sunlight, lamps, humidity and watering inside the greenhouse.

(a)	Identify one other component that is required for an automated system.	[1]

Microprocessor(s) / microcontroller(s). (1)

(b)	State two types of sensors that may be used in the automated system.	[2]

Thermometer / thermocouple / thermistor. (1)
Humidity sensor. (1)
Light sensor / light dependent resistor / LDR. (1)

(c)	Explain one way that actuators could be used in the automated system.	[2]

Actuators can open or close windows (1) depending on the temperature in the greenhouse (1).
Actuators can open or close valves (1) to water the soil (1).
Actuators can open or close blinds (1) to reduce the temperature inside the greenhouse (1).
Actuators can turn on the lamps (1) to increase the amount of sunlight given to the plants (1).
Or any other reasonable use of actuators within this type of automated system.

2.	A new house is being built and will include an automated system that will control the lights. All lights will be connected to Wi-Fi and users will be able to control them via apps. Lights can either be turned on and off manually or set to turn on and off automatically.

(a)	Explain one advantage of using an automated system to control the lighting.	[2]

If a user leaves the property, they can turn all lights on/off at once (1) which saves time / reduces the energy usage (1).
Lights can be set to turn on before the user arrives home (1) making the home more appealing when they arrive / improving security (1).
Users don’t need to move to turn lights on/off (1) which is more convenient than having to get up / move around (1).
Or any other appropriate advantage.

(b)	Explain one disadvantage of using an automated system to control the lighting.	[2]

Connecting each light to Wi-Fi requires energy (1) so the system will be less environmentally friendly (1).
If the Wi-Fi hub/router/access point fails (1) then all the lights will no longer be controllable (1).
More components are required to make the lights (1) which means more raw materials will be used / a worse impact on the environment (1).

3.	One characteristic of robotics is that they are programmable.

(a)	State two other characteristics of robotics.	[2]

A mechanical structure / framework. (1)
Electrical components / sensors / microprocessors / actuators. (1)

Companies are beginning to create self-driving cars.

(b)	Explain two advantages of using self-driving cars.	[4]

The cars can react faster to events (1) so (in the future) could be safer than human drivers (1).
Technical jobs are created (1) to develop the hardware/software/systems (1).
More independence can be given to elderly / those who can’t drive (1) as they no longer need a driving licence / can be driven by the robot/self-driving system (1).
Reduced traffic jams (1) as computers can have cars travel closer together (1).
Or any other reasonable advantages.

(c)	Explain two disadvantages of using self-driving cars.	[4]

If there is are any errors in the programs (1) then crashes/injuries could be caused (1).
The technology costs a lot to develop (1) leading to higher car prices (1).
Some people enjoy driving (1) and this past time will be lost (1).
Some jobs will be lost (1) such as taxi drivers / lorry drivers (1).
Or any other reasonable disadvantages.

(d) State two roles that domestic robots can perform. 	[2]

Robotic vacuum cleaner. (1)
Robotic lawn mower. (1)
Robotic kitchen. (1)

Accept other reasonable areas that domestic robots are able to perform.
Do not accept areas that are typically thought of as embedded systems rather than robots (e.g., microwave, dishwasher, washing machine).

4.	One characteristic of AI is the ability to reason.

(a)	State two other characteristics of AI.	[2]

The ability to learn / adapt. (1)
The collection of data. (1)
Rules for using the data. (1)

(b)	One type of AI system is known as an expert system.

Describe how an expert system works.	[3]

(A set of facts are put into a) knowledge base. (1)
(A set of rules are given in a) rule base (1).
An inference engine (1) is used to deduce answers to questions / carry out deductive reasoning (1).
An interface is what is shown to users (1) and is used to ask questions / give answers / the user to enter answers (1).
Accept other accurate descriptions of an expert system such as:
The inference engine can carry out forward chaining (1) where questions are asked resulting in an answer (1) / The inference engine can carry out backward chaining (1) where the answer is given and the inference engine works out what conditions are necessary to reach the answer (1).

(c)	One other type of AI system is machine learning.

Describe how machine learning works.	[4]

Machine learning is where a program automatically learns (1) by creating its own rules / adapting its own processes/data (1).
Training data (1) is fed into the system so that it can learn to identify certain patterns/features/rules (1).
The larger the training data set, the more accurate the system will become (1).
Weightings can be given as to how important different rules are. (1)
When given new data to analyse, the program can use the rules it created to give a probability that it satisfies some criteria / matches a question (1).

Award up to 3 marks for any real example of how machine learning works.

E.g. A set of recipes could be given for a particular type of meal along with a rating (1) (training data). These would be analysed to produce the processes/data/rules required. (1) When a new recipe is added, the program could analyse it to produce a rating. (1)

[Total 30 marks]