Lesson plan

Topic 5 Secondary storage

Objectives: Understand the need for secondary storage Describe the operation of the following types of storage and give examples of each: Magnetic storage Optical storage Solid-state storage

Content

Starter PowerPoint Guide: Topic 5 Secondary storage Students should be aware that secondary storage is needed to be able to permanently store data. When the power is turned off, RAM will lose the data stored in it – it is volatile memory. ROM would be an appropriate answer to the question on the starter as it is non-volatile. The obvious answers indicated by the slide are solid state drives. Main Secondary storage Take students through the definition of secondary storage and ask them to name uses for it. A selection of possible answers is given on the Answers slide. Storage methods Students are expected to know the three methods of storage given. Magnetic covers any type of drive or media that uses magnetism to store data. Hard disks are the most commonly used today, but tape drives are still used for backups. You may have an old floppy disk or zip disk that you can show students as an example of magnetic removable media. Optical disks include Blu-ray, DVD and CD. Solid state includes SSD hard drives, flash drives and SD cards. Mobile technologies such as tablets and smartphones use solid state to store data internally and for media (with SD cards). Other technologies are being considered for the future. These include storing data on DNA. One DNA sequence can hold about 1.5 GB of data. Hand out Worksheet 4 and ask students to complete Task 1. Topic 4 Worksheet 4 Magnetic disks Take students through the slides showing how magnetic disks work. In particular, make sure that students are aware of the internal operation of each different type of technology. If you can get hold of an old hard drive from the school technicians, it can be an interesting exercise to open it up in front of the class so that they see at first-hand the platters and read/write head inside. Identify the parts of a hard disk drive – the platter (disk) and the read/write arm. Explain how a disk is divided into tracks and sectors. Data is created by magnetising fine iron (ferrous) particles on the disk. North and South polarities represent 0s. Show students the video ‘How a hard disk works’ [0m00s-4m30s] https://www.youtube.com/watch?v=NtPc0jI21i0 This shows a hard disk developer at Seagate Technology along with the key features that are given on the slides. Students may also be interested in a more general similarly labelled video of how hard disks have developed over the years. ‘How do hard drives work’ [5m11s] https://www.youtube.com/watch?v=wteUW2sL7bc Optical storage Lasers are used to read optical storage devices such as CDs, DVDs and Blu-ray disks. Small ‘pits’ are burned into the reflective surface of a disk creating a dip, as opposed to a ‘land’. Light from the laser is shone on the surface and if it is reflected back by a pit or a land. The start and end of the dips is where pits change into lands or vice-versa. This is what is used to encode data. The surface of a CD as viewed under an electron microscope is shown on the slide. This shows each of the pits and lands that are burnt into the surface. Ask students how a DVD can store more data if the process is the same? (Answer: lasers are used with a smaller wavelength, so they can read smaller pits and lands this means that more of them can be fitted on a disc.) If anyone asks how a rewritable CD works, this is done using heat sensitive ink that changes between opaque and transparent states with a laser. Opaque areas cover the reflective layer of the CD behind the ink and are read as 1s. Interesting fact: a human hair is about the width of 50 tracks on a CD. The laser that reads a CD is red. Blu-rays are often thought to be read by a ‘blue’ laser, however, the actual colour is violet. This wavelength of light is smaller; hence it is possible for more pits and lands to be placed on a Blu-ray and therefore for more information to be stored. Ask students to complete Task 2 and Task 3 on Worksheet 5. Solid state drives (SSD) SSDs use non-volatile flash memory to store information. Flash memory is a type of randomly accessed memory, but unlike RAM it is not directly accessible by the CPU. It is solid, fast and durable and has no moving parts, unlike a magnetic hard disk drive. Briefly show students how flash memory works then focus on the advantages and disadvantages of the technology as secondary storage. The specification expects students to understand how floating gates and control gates are used to store data in flash memory. The specification also expects students to be aware of NOR and NAND types of flash memory. Two videos are provided to help students understand the technology. These are slightly harder than other videos, but match closely the expectations on the specification. How does Flash Memory work [0m0s-6m20s] https://www.youtube.com/watch?v=r2KaVfSH884 Floating gate technology with NAND transistors [1m43s] https://www.youtube.com/watch?v=AO7CNNZmtTw Go through the remaining slides on Flash memory that complement the videos. Make sure that students know that there are two different types of flash memory. Almost all students will have experienced NAND memory used for storage in USB sticks, SD cards and smartphones. Explain to students that the ROM chips on their desktop computers can also have their software (usually called firmware) updated. This is stored on NOR flash which can be directly accessed by the CPU. Ask students to complete Task 4 on Worksheet 5. Plenary The plenary gives students a chance to test each other on questions that cover the key concepts of the lesson. It is suggested that each student tackles two questions and asks two questions. Answers are given on the following slide. Hand out Homework 5. Topic 5 Homework 5 Topic 5 Homework 5 Answers