# Answers

Task 1

1.	The performance of a CPU is dictated and dependent on a number of factors. Complete the table below, stating how each of the factors mentioned affects the performance of a CPU.

| Factor | How this affects performance |
| --- | --- |
| Clock speed | Every CPU operation starts on the clock pulse. The higher the clock speed (Hz), the more instructions the CPU will be able to carry out per second. |
| Cache Size | Data and instructions used once are often needed again soon after and will be stored in cache. The more cache memory a CPU has available, the more likely it is to be accessed from (much faster) cache rather than revisiting RAM for the data. Cache is quicker than RAM, especially L1. |
| Number of cores | The more cores a CPU has available, the more duplicate processing units are available to carry out the instructions. If many tasks are running simultaneously, multiple cores will result in faster execution. If only one program is running, the processor may not be able to take advantage of multiple cores |

2.	Two computers have different specifications.

Computer A has a quad core processor with a frequency of 2 GHz.

Computer B has a dual core processor with a frequency of 4 GHz.

(a)	What is the maximum number of fetch-execute cycles that each processor could achieve?

Computer A: 4 cores * 2 GHz = 8 billion cycles per second

Computer B: 2 cores * 4 GHz = 8 billion cycles per second

(b)	Explain why choosing processor B may be the better choice.

The processor operates at a faster frequency/clock speed. This means that if the software only makes use of one core, in this case, it will work twice as fast.

(c)	Discuss how the use of cache can improve the performance of CPUs.

There are three types of cache – Level 1, Level 2, Level 3. The fastest is Level 1 and the slowest is Level 3. The faster cache is more expensive, so a CPU will have less Level 1 than Level 2 cache. If the processing unit needs data from RAM, it first requests it from Level 1 cache. If it finds it, there is a ‘hit’ and this is far faster than it being requested from RAM. If there is a ‘miss’, then it is requested from Level 2 cache, and so on for Level 3 and finally RAM. If a suitable amount of L1, L2 and L3 cache is provided then the CPU will have considerable performance gains as it rarely needs to access data from (slow) RAM, but can instead access it from fast cache.

For this reason, the performance of a CPU needs to be tested with real data that simulates computer use, as a CPU with more cache may outperform one with a faster clock speed / more cores.

Task 2

Match up the terms on the left with the statements on the right.

| Data Bus |  | Co-ordinates and controls all operations carried out by the computer. |
| --- | --- | --- |
| Control Bus |  | Used for communicating the physical addresses of computer memory elements/locations that the CPU wants to access. |
| Address Bus |  | Responsible for arithmetic operations and logical operations. |
| System bus |  | Responsible for transporting the actual physical data, the bits and bytes of information between memory and the CPU. |
| Control Unit |  | A set of registers which act as a working area, often storing numbers and results of calculations. |
| Clock |  | Carries command and control signals to and from every single connected device. |
| ALU |  | Controls the timing of the processor. |
| General Purpose Registers |  | The collective name for the data bus, control bus and address bus. |

Task 3

Label the boxes with the following components: CPU, keyboard controller, display controller.

Draw the connections between the Address bus and the components, showing the direction of each connection.