Answers

1.	An assembly program contains the following instruction:

MOV ACC, 053

This instruction moves the value in memory address 053 into a register.

(a)	State the opcode in this instruction.	[1]

MOV

(b)	State the name of the register used in this instruction.	[1]

Accumulator / ACC (1)

(c)	Name the two operands used in this instruction.	[2]

ACC/accumulator/register (1), 053/memory address (1)

(d)	When the instruction is executed, describe the stages, CPU components and registers that are involved in executing the instruction on the CPU. 	[6]

The program counter / PC will have the memory address of the instruction (1), the address will be copied to the memory address register / MAR (1), the memory location will be sent through the address bus (1) and the instruction then fetched from RAM (1) through the data bus (1). The instruction will then be sent to the current instruction register (CIR) (1). The opcode will be decoded by the control unit (1) and the instruction will be executed. (1) The value in 53 will be passed to the accumulator register (1). The PC / program counter will be incremented (1).

Accept descriptions that show the registers and processes involved in fetching the data stored in memory location 053.

2.	A child’s plastic drum kit toy contains an embedded system.

(a)	Explain what is meant by an ‘embedded system’.	[2]

A computer system (1) dedicated to a specific task (1) that is typically held in ROM / non-volatile memory (1). An embedded system can be part of a larger mechanical or electrical system (1).

Any 2 marks from the above.

(b)	Embedded systems are used as part of security systems. Explain why most home security alarms make use of embedded systems.	[3]

Answers may include:

They have input devices such as passive infra-red detectors / magnetic detectors (1)
They have a siren which is an output device (1).
The main unit has a microprocessor (1) which is used interpret the input device readings and decide whether to ring the alarm (1).
Accept other examples of input devices, output devices, storage and processing that are used within a typical security system.

[Total 15 marks]