Answers

Task 1

1.	Research on the Internet new computers from a manufacturer of your choice.

(a)	What is the typical amount of RAM and hard drive size that they are including in their computers as standard?

The amount of RAM included in a computer will depend on the cost and manufacturer. Typically, computers that are running MacOS or Windows will be supplied with at least 8GB to function with the typical software used. Many typical configurations will have 8, 16 or 32 GB.

The amount of hard drive space will depend on the amount spent and manufacturer along with whether it is a solid-state drive (SSD) or magnetic disk.

Typical sizes are from 128GB (low cost SSD) to 4TB (magnetic disk or very high cost SSD).

(b)	The amount of storage in hard drives is usually far higher than the amount of RAM on the computer.

Why doesn’t the CPU access programs and other data from the hard drive directly?

Hard drives have a slow access speed compared to RAM.

Magnetic hard disks are especially slow as they need to move an arm with the drive head on to the correct location for the data.

RAM is far faster – so for programs and data currently being used this makes the computer perform better.

Most the data stored on hard drives will not be required for the user in the near future.

(c)	RAM is volatile. Explain what this means.

The data stored in RAM will be lost when it has no power.

Task 2

1.	Complete the table below with as much detail as possible to show that you understand the main differences between RAM and ROM.

| Question | RAM | ROM |
| --- | --- | --- |
| What does it stand for? | Random Access Memory | Read Only Memory |
| What is often stored in it? | Operating system Running programs Data currently being used | Computing bootup instructions (Bootstrap loader) BIOS |
| What happens to its contents when you turn the power off? | They are lost This type of memory is volatile | They are retained This type of memory is non-volatile |
| Can you read from and write data to it? | Read and Write | Read Only. The contents are written to it permanently at the point of manufacture. (Modern ROMs will allow updates to occasionally be stored on them, but these are still thought of as ROMs as this is a very slow process that is rarely used.) |

2.	(a)	Explain why increasing the amount of RAM will improve the performance of a PC.

When a program is executing, the operating system copies instructions and data from secondary storage (e.g. hard drive) into RAM. The CPU can fetch them much faster from RAM than from main storage.

Having lots of RAM means that many currently running programs can be held in RAM at the same time and the CPU can access them very quickly, instead of having to swap programs out to main storage and back to RAM when the next instruction needs to be executed.

(b)	When a computer systems’ RAM becomes full it can use virtual memory.

Explain what happens if the computer does not have any virtual memory.

If there is no virtual memory then the computer cannot store any more data in RAM. This means that no more programs or documents can be opened. Indeed, it is possible that programs will start to crash as they will not be able to store more data.

(c)	Describe how virtual memory works in a computer system.

Virtual memory is an area on the hard disk which is used as an extension of RAM. When the operating system runs out of RAM in which to store the next instructions needed for a currently running program, it swaps out another program or data from RAM to virtual memory on hard disk. This frees up RAM for a new program or data to use. If the old program or data is needed, then these will be swapped with something else in RAM. This process is far slower than if more RAM were available to be used.