Answers

1.	State the purpose of the CPU. 	[1]

The CPU processes instructions and data that are input into the computer (1) (so that the result can be output).

2.	State two types of operation carried out by the ALU.	[2]

Arithmetic operations (1).

Logic (comparison) operations (1).

3.	Describe the steps involved in the Fetch-Decode-Execute cycle. 	[4]

The Processor fetches the next instruction to be executed (1) from memory.

The address of the next instruction is stored in the Program Counter / PC (1).
The Program counter / PC is then stored in the MAR (1).

The Program counter / PC is incremented (1).

The instruction (that is fetched from memory) is stored in the MDR / Memory Data Register (1).

The instruction goes from MDR / Memory Data Register to the CIR / Current Instruction Register (1)

The instruction is decoded (1).

The instruction is executed (1).

And the cycle begins again (1).

4.	Explain what the MAR is used for. 	[2]

The MAR / memory address register holds the address of the instruction/data (1) that is to be fetched from memory/RAM (1).

5.	Explain how the following can affect the performance of a CPU:

(a)	clock speed	[2]

The higher the clock speed, the more instructions can be executed per second (1) as every operation starts on a clock pulse (1). This improves the performance (1).

(b)	number of cores	[2]

The more cores there are, the more processors there are processing data at the same time (1).

More cores will improve performance (1) in some circumstances, but the computer system may not always be able to take advantage of more than one core. (1)

6.	(a)	Define what is meant by an embedded system.	[1]

A computer system which is built into another device (1).

(b)	Give three examples of devices which use embedded systems.	[3]

Washing machine (or any other domestic appliances that have embedded systems), cars, security systems, lighting systems, vending machines. (Many other answers acceptable).

7.	(a)	State two differences between RAM and ROM. 	[2]

RAM is volatile whereas ROM is non-volatile (1).  RAM is read/write whereas ROM is read only (1).

(b)	List two items typically held in RAM when a computer is running. 	[2]

The operating system (1) / or the part of the operating system currently in use (1).

The programs / software currently in use (1).

The data in use by those programs (1).

(c)	Give two uses for ROM.	[2]

Storing bootup instructions (1).

Storing programs in embedded systems (1).
Storing the BIOS (1).
Accept: Storing computer games in ROM cartridges (must make clear that it is ROM cartridges or older consoles).

8.	Describe the use of cache memory and the role it plays in improving processing speeds.	[3]

Small area of memory which is either on the CPU or between the CPU and RAM (1).

Purpose is to store the most frequently accessed / recently accessed instructions and data needed by the processor (1). 
Cache is faster than RAM (1).

If data is in the cache the processor can access it faster than RAM (1).

If an instruction is not located in the cache the processor has to retrieve it from RAM (1).

9.	(a)	Explain what is meant by virtual memory and why it is needed.	[3]

Virtual memory is a reserved/allocated area of disk (1) used as an extension of RAM (1).

When RAM becomes full of programs and/or data, instructions /data not currently being used, these are swapped to virtual memory (1) This frees area in RAM for new programs or data (1).

When the old instructions/data are needed, something else is swapped out (1) and the needed instructions/data swapped back into RAM (1).

Data is stored in areas of virtual memory known as pages (1).

(b)	Tom’s computer is running slowly, so he decides to install more RAM. Explain 
how this will affect the performance of his computer. 	[2]

Adding more RAM will mean there will be fewer data transfers to virtual memory (1).

As virtual memory will be used less often (1) this will lead to improved performance (1).

10.	(a)	Explain why computers need secondary storage. 	[2]

RAM is volatile so all data is lost when power is switched off (1).

Secondary storage is required for permanent storage of software and data / it is non-volatile (1).

(b)	Optical storage makes use of a laser to read a disk. One type of optical storage is a compact disc (CD). Name one other type of optical storage. 	[1]

Blu-ray. (1)
DVD / Digital Versatile Disc / Digital Video Disc. (1)

One storage technology is solid state flash memory.

(c)	State which type of flash memory is normally used for a solid-state hard drive that is used to store user files.	[1]

NAND (flash memory) (1)

(d)	Describe how one bit of data is stored in solid state memory. 	[3]

Electrons are stored inside a floating gate (1) which has an insulation layer / silicon dioxide layer above and below it (1). If electrons are present, current is unable to flow (1) resulting in a 0 being stored (1). If no electrons are present in the floating gate, current can flow, resulting in a 1 being stored (1). By applying a high voltage / negative voltage to the control gate (1) electrons can be forced in or out of the floating gate (1).

(e)	Explain one reason why solid-state storage is used in digital cameras and smartphones.	[2]

Solid-state storage has no moving parts (1) so is more reliable in portable devices (1).
Solid-state storage requires less space per unit of data (1) than optical or magnetic devices (1).

[Total 40 marks]