# Answers

Task 1

Match the words on the left to the meanings on the right.

| Network packet terminology |  | Meaning |
| --- | --- | --- |
| Header |  | The particular computer or server that the packet is being sent to. |
| Destination address |  | Used to determine the place of this packet of data in the overall order. |
| Originating address |  | This is located at the start of the packet and contains the destination address, originating address and packet number. |
| Packet number |  | Also known as the sender. This is the computer or server that the packet has come from. |
| Payload |  | This contains information that shows it is the end of the packet. It may also contain data that can be used to check if there is an error in the packet. |
| Trailer |  | The actual contents of the packet. |

Task 2

The following table shows eight packets of data that are received by a home router.

| Destination address | Originating address | Packet number | Payload | Trailer |
| --- | --- | --- | --- | --- |
| 192.168.1.2 | 192.168.1.1 | 4 | 01001011 | 4 |
| 192.168.1.2 | 192.168.1.1 | 3 | 01100011 | 3 |
| 192.168.1.2 | 192.168.1.1 | 1 | 01010000 | 2 |
| 192.168.1.2 | 192.168.1.1 | 5 | 01000101 | 3 |
| 192.168.1.2 | 192.168.1.1 | 2 | 01000001 | 2 |
| 192.168.1.3 | 192.168.1.1 | 3 | 01011010 | 4 |
| 192.168.1.2 | 192.168.1.1 | 3 | 01000011 | 3 |
| 192.168.1.2 | 192.168.1.1 | 6 | 01010100 | 3 |

(a)	The trailer inside each packet contains a checksum. The checksum is calculated by adding the total number of 1s in the payload.

Cross through the packet of data which has an error in it.

The second row has packet 3 incorrectly transmitted. The third bit (working left to right) should have been a 0, as can be seen when the packet is re-transmitted in the second from bottom row.

The checksum more likely to be used is known as the cyclic redundancy check (CRC) and more complicated than this example.

Remind students that all other parts of the packets (destination address, originating address, packet number and trailer) would also be in binary in an actual packet of data.

(b)	A message has been sent to the computer connected at IP address 192.168.1.2.

The payload in each packet shows one character in ASCII.

Workout each letter, then re-order the packets to work out the word that has been transmitted. Your teacher may give you a copy of an ASCII table to help you, or you can make use of an Internet search to find one online.

Please see the file: ASCII Codes.docx for a copy of an ASCII table to provide students.

| 1 | 01010000 | P |
| --- | --- | --- |
| 2 | 01000001 | A |
| 3 | 01000011 | C |
| 4 | 01001011 | K |
| 5 | 01000101 | E |
| 6 | 01010100 | T |

Word transmitted: PACKET

Note that one packet has the destination of 192.168.1.3 and should be ignored as the message we are looking for is that sent to 192.168.1.2