Answers

1.	Javid is looking to set up a home network and is becoming puzzled by some of the data transmission terminology.

Draw a line to match each description to the appropriate technical term.	[3]

| Simplex |  | Data travels in both directions simultaneously |
| --- | --- | --- |
| Half-duplex |  | Data travels in one direction only |
| Full duplex |  | Data travels in both directions, but not simultaneously |

2.	A cable contains eight wires that are used for the transmission of data.

(a)	State whether the cable is serial or parallel.	[1]

Parallel

(b)	One problem that the method of data transfer will have is skew.

Explain what is meant by skew in data transfer.	[2]

The signals/bits sent through each cable will arrive at slightly different times.(1)
The receiver needs to wait until all bits/signals have arrived on all the lines before the next set of bits can be sent. (1)
This slows down data transfer (1).

(c)	It is also possible that the signal sent through one wire may interfere with the signal sent through another wire. This may cause an error in the transmitted data.

State the name for this problem.	[1]

Crosstalk. (1)

(d)	State a method of transmission that will have neither of the problems that are 
given in parts (b) and (c).	[1]

Serial transmission.

(e)	A CPU communicates with RAM using parallel transmission.

Describe the reasons why parallel transmission is used over serial transmission 
in this case.  	[2]

RAM and CPU are located very close together on a motherboard (1) which means they don’t suffer very much from skewing. (1)
The lines on the CPU/RAM and motherboard are all hard wired with enough distance between each connection so that they don’t suffer from crosstalk. (1) (In a cable the wires may be located very close together over a long distance which causes crosstalk).
Large amounts of data need to be transmitted between CPU and RAM (1) so by using parallel transmission 32-bits/64-bits of data / the contents of one register can be transmitted in one clock tick.

(f)	Explain whether simplex, half-duplex or full duplex is used for transmitting data between RAM and CPU.	[2]

Half-duplex (1) – the data is either transmitted between RAM and CPU, or the other way around, but never at the same time (1)

[Total 12 Marks]