# Answers

Task 1

Explain how, in some cases, it is possible to send data more quickly to a printer through a parallel cable rather than a serial cable.

The parallel cable has 8 lines for data meaning that one byte (8 bits) of data may be sent at the same time.

The serial cable has one line for sending data (The transmit data / TXD lines), so can only send 1 bit of data at a time.

Assuming that the rate the data is sent through each wire/line is the same, the parallel cable will be able to send data at 8 times the speed.

Task 2

(a)	Explain the problem known as ‘skew’ with parallel transmission.

Parallel wires all have different properties, which affect the speed at which signals 
(bits) travel. This means each bit arrives out of sync.

The receiver needs to wait until all data has arrived for each line, before the next bits of data can be sent.

(b)	USB cables are commonly used to connect peripheral devices to computers.

(i)	Explain how it is possible for USB cables, which perform serial transmission, to be faster than traditional parallel cables that were used to communicate with printers in the 1990s.

The parallel cables are able to send 8 lines of data at the same time and therefore 8 times the data of USB cables (which only have one line for sending data).

However, the speed that data is sent through the one line in USB is significantly faster. This allows for more data to be sent through one line, than all 8 lines combined in a parallel cable.

(ii)	Research USB cables and make notes of the advantages they have over the older parallel cables that they replace.

They can be plugged in / taken out without having to turn off the computer.
The sockets used are smaller.
The pins used on the older cables were easily bent which broke the cable (or they had to be bent back). USB uses contacts instead of pins.
USB has a 5V line, it is therefore possible for lower power devices to work without a separate power cable.

Task 3

(a)	Explain what is meant by the following terms:

(i)	Serial, duplex transmission: Data is sent both ways simultaneously down single cables. (Two cables are actually required, but each behaves serially.)

(ii)	Parallel, half-duplex transmission: Data is sent, several bits at a time, down multiple wires, and can be sent back through the same multiple wires. However, sending and receiving cannot happen simultaneously.

(iii)	Serial, simplex transmission: Data is sent down a single cable in one direction only. The data is sent one bit at a time.

(b)	Many homes in the UK are connected to the Internet using ADSL (Asymmetric Digital Subscriber Line).

Research this type of connection and answer the questions that follow:

(i)	Why is the line known as asymmetric?

The line allows the computer to upload and download at the same time. It is known as an asymmetric connection as the download speed (bandwidth) is greater than the upload speed (bandwidth).

Another technical reason is that crosstalk is more likely at the cabinet where all the customer connections are close to each other. In the home, other customer connections won’t have any crosstalk.

As customers typically require more bandwidth for download than upload this solution has both technical benefits along with a better allocation for use of the bandwidth.

(ii)	Is the line duplex, half-duplex or simplex?

It is a full duplex line as both uploading and downloading of data can happen at the same time.