Lesson plan

Topic 1 Manual input devices

Objectives: Understand what is meant by an input device Understand each of the following input devices and why they are required Keyboard Microphone Optical mouse Touch screen (resistive, capacitive and infra-red)

Content

Starter PowerPoint Guide: Topic 1 Manual input devices The starter gives students an opportunity to consider the devices that they use in school and at home. Ask them what input devices are used in these devices. Some answers will be simple and generic, others such as games controllers will commonly be used. Whilst many of the examples (such as games controllers) are not on the specification, they are useful to elicit from students as they show an understanding of what input controllers are. Main Computer systems Make students aware of the overall structure of computer systems. Computer systems will always require a primary storage (typically RAM for working memory). They are also likely to need secondary non-volatile storage. Keyboards and keypads Keyboards and keypads are probably the most common form of data entry into a computer. They have the advantage of being easy to use and also require very little training. Specialist keyboards such as numeric keypads, ergonomic keyboards or gaming keyboards, have also been produced to suit various applications. The main drawbacks are that it is very easy to make typing errors and data entry is also very time consuming even if the user is able to touch-type. Keyboards are connected to the computer either by using a USB port or a wireless dongle. Tablets and mobile phones usually use some form of touchscreen virtual keyboard which can be even slower or more error-prone when entering data. How do keyboards work? It is important to note that the specification does not require that students understand the inner working of devices. In some cases we have kept these explanations in the topics as they are likely to be of interest to students. They will also help students understand whether devices will be suitable to a given context, which is a requirement of the specification. It would be useful here to have an old keyboard so that the students can see the inner workings of the device. Inspection of underneath the keys on a typical keyboard shows small holes in a plastic box; the keys have a small plastic peg which is the same shape and size as the hole. The keyboard consists of three layers of plastic: two layers are covered with electrical tracks and the middle layer is an insulating layer containing holes through which the pegs on the keys pass. When a key is pressed, the peg pushes through the hole and makes contact with a membrane containing electrical tracks. This completes a circuit and a signal is sent to the computer. The computer interprets this signal and works out which key was pressed. Pieces of rubber under a key make it bounce back to its original position so it can be pressed again. When the key bounces back, the electrical connection is also broken. Optical mouse A mouse can be mechanical or optical. Only optical mice are mentioned on the specification. An optical mouse shines a red LED onto a surface with the reflection being detected by a sensor to detect movement. A sensor takes a low-resolution image of the surface. As the mouse moves, images are continuously taken and compared with the previous image to calculate the direction of movement. Microphones A microphone consists of a diaphragm, a coil and a permanent magnet. When sound reaches the diaphragm, it vibrates backwards and forwards. This causes a coil to move backwards and forwards at the same time. As the coil moves, it changes the magnetic field produced by the permanent magnet. This change creates an electric current (in analogue form) which needs to be converted into a digital signal so the computer can understand the data being sent to it by the microphone. The ADC carries out this conversion. Once converted into digital, the computer can store or process this data. Hand out Worksheet 1 and ask students to complete Task 1. Topic 1 Worksheet 1 Topic 1 Worksheet 1 Answers Touch screens on tablets and mobile phones. There are four common technologies currently used by tablet and mobile phone manufacturers (the technologies are also used in a number of other applications): - Resistive - Capacitive - Infra-red This topic explains how each of the technologies work as they are listed separately on the specification, suggesting that students would need to understand the technological differences in order to choose the correct screen for a given context. Resistive Touchscreens using resistive technology are made of an upper layer of conducting polymer (usually polyester coated in indium tin oxide – ITO). There is also a bottom layer which made from glass with a conductive coating (using ITO). There is an insulating layer sandwiched between these two conductive layers. When the screen is pushed (by a finger or stylus), the two conducting layers make contact and complete an electric circuit. The position where the screen is touched is calculated by a microprocessor which interprets the signals sent out when the screen is touched. Strengths: - relatively inexpensive technology - can be used with bare finger, gloved hand or stylus Weaknesses: - not so good in strong sunlight; image sometimes poor relatively easy to scratch the polymer screen - can only support single touch (doesn’t support multi-touch functions) Capacitive All modern smartphones use capacitive touchscreens. Touchscreens using capacitive technology are made up of multiple layers of thin glass. The bottom of the first layer and top of the last layer of the glass are coated in ITO to make the surface conductive. There is an insulating layer of glass between these two conductive layers – this makes the screen act like a capacitor. When a finger touches the screen, the electrical field changes; the coordinates where the screen was touched is worked out by a microprocessor. Unlike resistive, capacitive technology allows multiple touches (enabling ‘pinching and zooming’ on a mobile phone and so on). Strengths: - supports multiple touch facility - has good contrast in strong sunlight - very durable technology Weaknesses: - can only be used with a bare finger or with capacitive pen - relatively expensive technology Infra-red Touchscreens using infra-red technology use a grid pattern of LEDs and photocells (light sensors). The LEDs shine IR light in front of the screen in a grid pattern. When the screen is touched the IR beams are interrupted – this indicates the coordinates where the screen was touched. A microprocessor calculates these coordinates and sends the information to the device. These are commonly used in fast food restaurant tills as the screen can then be coated in grease and scratch resistant rubber. Strengths: - good contrast in strong sunlight - very durable technology - can be used with bare finger, gloved hand or stylus Weaknesses: - ambient light can sometimes cause problems if screen is scratched - only supports single touch (doesn’t support multi-functions) Ask students to complete Task 2 on Worksheet 1. Plenary Ask students to discuss the answers to the questions on the slide in pairs. Once they have had time to consider their answers, ask pairs to feed back their answers. Hand out Homework 1. Topic 1 Homework 1 Topic 1 Homework 1 Answers