Lesson plan

Topic 5 Printers

Objectives: Understand each of the following printing devices and why they are required Inkjet printer Laser printer 3D printer

Content

Starter PowerPoint Guide: Topic 5 Printers The starter slide shows a large inkjet printer. Ask students to name different technologies that are used in printers. It’s likely that they will have seen or had experience with inkjet, laser and 3D printers. They are also likely to have used thermal printers when getting till receipts. Other printers exist. For example, lithographic printers will create a plastic or metal plate (which is an expensive process), but from that plate, they will be able to create multiple high-quality copies very cheaply. This technique is used for books, magazines and brochures. Main - Inkjet printers - Demonstrate the internal parts of an old inkjet printer if you have one. Tell students that they contain cartridges of liquid ink, usually in four colours: cyan, magenta, yellow and black (CMYK). - The print head assembly contains nozzles which spray ink droplets onto the paper and a belt is used to move the print head across the paper. - Stepper motors are used to move the print head across the page and to feed paper into the printer. - How are the ink droplets produced? - There are two common methods of producing ink droplets to be sprayed on the paper using ink jet printers. The two methods are: Thermal bubble Piezoelectric crystal - The operation of both systems is quite different: - Thermal bubble - This produces droplets of ink using the following method: - behind the print nozzles there are tiny resistors that create heat (the temperature momentarily heats to over 300°C) - the heat causes the ink to instantly vaporise and form a tiny bubble - as this bubble expands, ink is ejected onto the paper - as it cools, the bubble collapses and draws in fresh ink - this process continues until the page is printed - a typical print head has over 300 tiny nozzles - Piezoelectric crystal - This produces droplets of ink using the following method: - a small piezoelectric crystal is located at the rear of each ink reservoir of each nozzle - when the crystal receives a small electric current, it begins to vibrate in and out - as it vibrates inwards, ink is ejected onto the paper - as it vibrates outwards, ink is drawn in again to replace the ink sprayed - Highlight the benefits and drawbacks of the two technologies. Hand out Worksheet 5 and ask students to complete Task 1. Topic 5 Worksheet 5 Topic 5 Worksheet 5 Answers - Laser printers - Laser printers use dry toner rather than liquid ink as in inkjet printers. They rely heavily on the use of electrostatic charges to produce text and images on paper. Unlike inkjet printers, they produce a whole page at a time rather than print line by line. Printing with laser printers is usually faster as a print-head is not required to move backwards and forwards across the page. Instead, a drum simply rotates as the page moves through. Laser printers can print in colour by using four colours of toner (CMYK). Ask students to complete Task 2 on Worksheet 5. 3D printers 3D printers are increasingly finding all kinds of uses. If students have one in school, ask them about their experiences and how it works. Show the videos: 3D printing – What is it? [1m15s] https://www.youtube.com/watch?v=EF8CNR-gcXo Hershey’s 3D printer prints chocolate [1m42s] https://www.youtube.com/watch?v=slksoDJPJ94 Take students through the slides to show how the printers work and how they can be used. In particular, highlight how useful 3D scanners can be in creating the 3D models that are used by 3D printers. Applications in medicine After going through the slide, play the video: 3D printed prosthetic hands [3m03s] https://www.youtube.com/watch?v=Cl8ijPGEKO8 Ask students to complete Task 2 on Worksheet 5.

Plenary The plenary gives students a chance to consider the different types of printer available, how they work and examples of where they are required. Remind them that the exam board may well give them a context and then expect them to choose the correct printer (or input/output devices). Go through some suggested answers on the following slide. Hand out Homework 5. Topic 5 Homework 5 Topic 5 Homework 5 Answers