# Answers

# Task 1

1.	A manufacturer of children’s toys wants to develop a toy that is aimed to keep young children, in the rear of the car, occupied on long journeys.

The toy will have the following features:

- Displays films and videos

- Plays music

- Has objects at the sides, such as squares, circles and triangles that move in and out in time with music

(a)	The device will need a display to show the films and videos.

State two other output devices that the toy will need.

Display
Speaker
Actuators

(b)	Choose one technology for the display and justify your choice.

Either choice is acceptable with the correct justification.

LCD display – cheaper to manufacture.
LED display – brighter; will last longer; more colour contrast/vivid colours.

# Task 2

1.	There are two types of technology that are used in projectors.

State what each technology is and describe how it works.

DLP projectors – use micro-mirrors on a microchip for each pixel. The mirrors tilt toward the light source to be turned on, and away to be turned off. A rotating disc changes the colour very quickly between red/green/blue. The mirror can, for example, turn on as blue passes, then off as red and green pass. This would create a blue pixel on the screen.

LCD projectors – use three LCD screens – one for red light, one for green light and one for blue light. The three sets of light are combined together using a prism.

2.	Research DLP and LCD projectors and complete the table below to show the advantages and disadvantages of each.

Advantages and disadvantages of DLP and LCD projectors.

| DLP advantages and disadvantages |  | LCD advantages and disadvantages |  |
| --- | --- | --- | --- |
| Sharp picture quality / higher contrast | Cheaper |  |
| Higher definition / more pixels | Quieter |  |
| Smoother video | Use less power and be quieter |  |
| Smaller | Dust filters need replacing |  |
| ‘rainbow’ effect for some people when viewed from an angle | Less heat output |  |
|  | No ‘rainbow effect’ |  |
|  | Brighter colours when 3LCDs are used |  |