Answers

1.	A deluxe dog kennel is being designed which will have the following features:

- A heat pad which turns on if the temperature is below 18oC

- A water bowl which refills if below a certain level

- A window fan which blows air into the kennel if the humidity is greater than 70%

- A digital camera which takes a photo if the dog barks

- A ‘Doggy home’ light which turns on if the dog is in the kennel

For each of the features, complete the table below to state a suitable sensor 
that could be used. 	[5]

| Feature | Appropriate sensor |
| --- | --- |
| A | Temperature sensor |
| B | Fill sensor Accept moisture sensor Do not accept humidity sensor |
| C | Humidity sensor Do not accept moisture sensor |
| D | Acoustic sensor |
| E | Pressure sensor Do not accept an infra-red sensor as this will not be triggered unless the dog is moving. |

2.	Screens are manufactured using different types of technology.

(a)	Most televisions and computer screens now use LCD technology. These are back-lit using LEDs or a fluorescent (CCFL) bulb.

(i)	State what each of the initials below stands for:	[2]

LCD: Liquid Crystal Display

LED: Light Emitting Diode

(ii)	Give three advantages of using LED technology rather than a fluorescent 
bulb for back lighting.	[2]

Reach maximum brightness almost immediately. (1)

Gives a brighter light / more lumens. (1)

Lasts longer. (1)

Lower power consumption. (1)

The display can be made thinner / lighter. (1)

A larger colour spectrum is available. (1)

(b)	Projectors are used in school classrooms, business presentations and cinemas.

State two technologies that are used for digital projectors.	[2]

Digital light processing / DLP projector. (1)
Liquid crystal display / LCD projector. (1)

3.	A library uses a barcode scanner to scan the barcode in a book when a library user borrows or returns the book.

(a)	Describe why a barcode scanner is required.	[4]

The scanner will read the barcode (1) and process it to give an ID number for the book. (1)
The library database will be updated (1) to decrease the quantity of the book title by one. (1)
The book ID number will be added to the borrower’s account. (1)
Scanning the barcode will be faster than entering the ID number by hand. (1)

(b)	Describe one other application that makes use of barcode scanners.	[2]

Supermarkets (1) for use in stock control/retrieval of prices (1)

Tracking packages (1) as they go through a distribution network / to show that they have been delivered to the customer (1).

Accept other applications with a description of how it is used.

4.	A restaurant decides to use QR codes so that customers are able to browse the menu on their phones.

(a)	Define a QR code.	[1]

Quick Response Code. (1)
A 3-dimensional bar code. (1)

(b)	Describe how a potential customer could use the QR codes to browse the menu.	[2]

Scan code with mobile phone/tablet. (1)

Use App on phone/tablet to decode the QR code. (1)

Visit the hyperlink that was encoded in the QR code. (1)

5.	A smartphone manufacturer decides to use a capacitive touch screen rather than a resistive touch screen on a new phone.

(a)	State one other technology which is used on touch screens.	[1]

Infra-red (touchscreen) (1)

(b)	Explain two benefits of using a capacitive touch screen rather than a resistive one.	[4]

A capacitive screen can detect multiple finger presses at the same time (1) 
allowing for gestures (such as pinch/swipe). (1)
A capacitive screen is more accurate (1) allowing for an easier selection of icons/menu items. (1)
A capacitive screen doesn’t need so much pressure to interact with (1) making it easier to use / feel higher quality (1).

6.	A steel manufacturing company uses a gas furnace to heat the steel to its required temperature.

The temperature of the furnace can be increased or decreased by controlling a gas supply valve. The whole process is controlled by a computer system which receives data from sensors in the furnace.

(a)	Name a suitable type of sensor for this application. 	[1]

Temperature sensor / Thermistor

(b)	Describe how the sensors and computer interact to control the temperature 
of the furnace. 	[4]

The temperature sensor reads how hot the furnace is (1) which is analogue data (1).
This is converted to a digital signal using an ADC / Analogue to Digital Converter (1).
If the temperature is less than a pre-stored value (1) then the computer sends a signal to the actuator to open the gas valve (1).

If the temperature is greater than the pre-stored value then the computer sends a signal to the actuator to close the valve (1).

7.	Complete the following table by naming a suitable sensor for the given application. 	[5]

| Sensor | Application |
| --- | --- |
| Infra-red / pressure / acoustic / magnetic | Detection of an intruder in a burglar alarm system. |
| pH | Monitor the acidity levels in a chemical manufacturing plant. |
| Infra-red | Automatically turn on the lights in a school corridor if someone is walking down it. |
| Accelerometer | Determine if a smartphone is orientated in landscape or portrait. |
| Gas / CO / CO2 / O2 / NOx | Monitoring air pollution levels at the side of a main road. |

8.	A school has just held a musical and decided to give everyone who took part a photo of the cast and crew.

They decide to use an inkjet printer to make the photos.

(a)	State one other type of printer that they could have used. 	[1]

Laser printer (1)

(b)	Describe how the inkjet printer works to print the photos.	[3]

The information is sent to the printer and stored in RAM/memory. (1)
An actuator moves a roller (1) which moves the paper through the printer. (1)
The page stops when it is under the print head. (1)
The print head moves across the page. (1)
Nozzles on the print head heat to form a bubble. (1)
This causes a fine drop of ink to be released from the nozzle onto the page. (1)
The page moves further so the print head can print the remaining lines, (1) until the page is released from the printer. (1)

9.	A manufacturer of handheld puzzles first makes their designs using clay.

(a)	Describe how a 3D scanner would be used to create a 3D model of the clay 
designs on a computer. 	[3]

The 3D scanner will use a laser beam to scan the object. (1)
Two cameras will be able to detect the laser beam (1) and using triangulation, they will be able to calculate where the beam is located (as an x,y,z coordinate). (1)
The camera will move around the object (1) so that scanner can build up a point cloud of the object. (1)
The points are sent to the computer so that the software can build the 3D model. (1)

(b)	The company will make changes to the 3D model before printing the designs using a 3D printer.

Describe how a 3D printer is able to print the model.	[3]

The 3D model is sliced into many 2D images. (1)
Plastic is fed to the print head. (1)
The print head prints the first layer of the 3D model. (1)
It then moves to each of the layers in turn (and prints them). (1)
This process is repeated until the whole 3D model has been printed. (1)

(c)	Once the model is complete it will be photographed so that the designers have a reference of the design. The photograph will be saved on a computer.

State an input device that would be used to take the photograph. 	[1]

A digital camera. (1)
Accept a camera if a 2D scanner is also given.

(d)	Other than prototype designs, give two other uses for 3D printers. 	[2]

Reconstructive surgery (1)
Prosthetic limbs (1)
Artistic pieces (1)
Spare parts (1)
Fashion products (1)
Food production (1)
House building (1)
Or any other reasonable use for 3D printing.

(e)	The designers of the puzzles will make changes to the 3D model before it is sent 
to the 3D printer.

Explain one input device they will use to make these changes.	[2]

A keyboard (1) to use shortcuts / to select menu items. (1)
A mouse / optical mouse (1) to select points on the model / make adjustments to the model / select menu items. (1)
A touch screen (1) to select / move / manipulate the model / select menu items. (1)
Or any other appropriate input device and use.

[Total 50 marks]