Copyright

© 2021 PG Online Limited

The contents of this unit are protected by copyright.

This unit and all the worksheets, PowerPoint presentations, teaching guides and other associated files distributed with it are supplied to you by PG Online Limited under licence and may be used and copied by you only in accordance with the terms of the licence agreement between you and PG Online Limited. Except as expressly permitted by the licence, no part of the materials distributed with this unit may be used, reproduced, stored in a retrieval system, or transmitted, in any form or by any means, electronic or otherwise, without the prior written permission of PG Online Limited.

Licence agreement

This is a legal agreement between you, the teaching institution, and PG Online Limited. PG Online Limited grants to you a non-exclusive, non-transferable, revocable licence to use this unit and all the worksheets, PowerPoint presentations, teaching guides and other associated files distributed with it in the course of teaching by your teachers and/or employees.

The materials distributed with this unit may be copied and used by your teachers and/or employees on a single site only in the course of their teaching. You warrant that you shall not, and shall procure that each of your teachers and/or employees shall not, share in any way any of the materials or part of the materials with any third party, including users on another site or individuals who are teachers and/or employees of a separate institution. You acknowledge and agree that the materials must remain with you, the teaching institution, and no part of the materials may be transferred to another institution. You also warrant that you shall not, and shall procure that each of your teachers and/or employees shall not, procure, authorise, encourage, facilitate or enable any third party to reproduce these materials in whole or in part without the prior permission of PG Online Limited.

In consideration of the licence granted to you, you shall indemnify PG Online Limited against all liabilities, costs, expenses, damages and losses (including but not limited to any direct, indirect or consequential losses, loss of profit, loss of reputation and all interest, penalties and legal costs and all other professional costs and expenses) suffered or incurred by PG Online Limited arising

Teacher’s Guide

Introduction

This teacher’s guide contains a detailed lesson plan to accompany the set of PowerPoint slides and worksheets for each topic.

The lesson plans are designed to form a basis for ideas for the teacher and should be adapted to suit the teaching style and preferences of the individual teacher, and the resources and nature of the individual school or Computer Science department.

The material supplied for this unit includes:

5 PowerPoint presentations, each designed to cover one topic, which may take more than one lesson

5 worksheets

5 homework sheets

An end-of-unit test for assessment purposes

Summary

The unit is subdivided into seven topics and an end-of-unit assessment. Each topic is designed to give enough materials for approximately 1 hour of teaching time, however, this will differ depending on how much depth is considered and whether homeworks are reviewed in the following lesson. The unit covers Section 3.2 of the Cambridge International iGCSE Computer Science 0478/0984 specification. Input devices, output devices, sensors and the selection of appropriate devices for given contexts are all covered.

The unit covers the large number of input and output devices that are required by the specification. All key input and output devices are introduced, with a special focus on the technologies that students may have less understanding of such as 3D scanners, actuators and various sensors such as accelerometer, infra-red and proximity sensors. Students will also be shown how to identify the type of data captured by each sensor. Finally, an end of unit final assessment test will cover all aspects of the unit.

Learning Outcomes for the unit

At the end of this Unit all students should be able to:

Understand what is meant by an input device

Understand each of the following input devices and why they are required

Keyboard

Microphone

Optical mouse

Touch screen (resistive, capacitive and infra-red)

Understand each of the following input devices and why they are required

Barcode scanner

QR code scanner

Digital camera

Two-dimensional (2D) scanners

Understand the purpose of the following sensors

Infra-red

Light

Pressure

Temperature

Understand what is meant by an output device

Understand each of the following output devices and why they are required

Speakers

- Most students will be able to:

Understand each of the following input devices and why they are required

Touch screen (resistive, capacitive and infra-red)

Understand each of the following input devices and why they are required

Three-dimensional (3D) scanners

Understand the meaning of a sensor

Understand the purpose of the following sensors

Flow

Gas

Humidity

Moisture

pH

Proximity

Understand each of the following output devices and why they are required

Speakers

Light emitting diode (LED) screens

Liquid crystal display (LCD) screens

Understand each of the following printing devices and why they are required

Inkjet printer

Laser printer

3D printer

Some students will be able to:

Understand the purpose of the following sensors

Acoustic

Accelerometer

Level

Magnetic field

Understand each of the following output devices and why they are required

Liquid crystal display (LCD) projectors

Digital light processing (DLP) projectors

Actuators

Previous Learning

- No prior knowledge is essential with this unit. However, students should have a basic understanding of computer systems from lessons delivered as part of the Key Stage 3 national curriculum.

Suggested Resources

No specific software is required for this unit. It is primarily a theoretical unit that can be taught entirely without computers, but there are some exercises where computers could be used to support understanding through practical discovery.

Recommended textbooks

Cambridge IGCSE Computer Science 0478 / 0984 SN978-1-910523-38-4

Experience + revision science + beautiful design = better results

Absolute clarity is the aim with a new generation of IGCSE Computer Science revision guide. This clear study guide has been expertly compiled and edited by successful former teachers of Computer Science, highly experienced examiners and a good dollop of scientific research into what makes revision most effective and more accessible.

Past examinations question are essential to good preparation, improving understanding and confidence. This guide has combined revision with tips and more practice questions than you could shake a stick at. All the essential ingredients for getting a grade you can be really proud of.

Each specification topic has been referenced and distilled into the key points to make in an examination for top marks. Questions on all topics assessing knowledge, application and analysis are all specifically and carefully devised throughout this book.

This book is suitable for the 0478 9-1 IGCSE, the 0984 A*-G IGCSE and the 2210 O Level for examination in 2023 onwards.

A new generation of revision book to capture students’ attention and make revision more effective and more enjoyable.

- Hundreds of marks worth of examination style questions

- Answers provided for all questions within the books

- Illustrated topics to improve memory and recall

- Specification references for every topic

- Examination tips and techniques

- Free Python solutions pack

Vocabulary

Vocabulary associated with this unit, such as:

Input device, barcode scanner, digital camera, keyboard, microphone, optical mouse, QR code scanner, touch screen, resistive, capacitive, infra-red, 2D scanner, 3D scanner, output device, actuator, digital light processing (DLP) projector, inkjet printer, laser printer, light emitting diode (LED) screen, liquid crystal display (LCD) projector, LCD screen, speaker, 3D printer, sensor, acoustic, accelerometer, flow, gas, humidity, infra-red, level, light, magnetic field, moisture, pH, pressure, proximity, temperature.

Assessment

Homework is given for each lesson. These consist of a mixture of short, factual questions assessing knowledge in isolation and longer questions in which students are asked to analyse a situation or justify their answer to questions.

The final assessment given assesses student’s knowledge of the current unit. In the actual examination, questions may integrate various topics into one question. As such, we strongly encourage teachers to use past papers and specimen papers to assess students and predict outcomes.

Lesson plans

Topic 1 Manual input devices

Objectives: Understand what is meant by an input device Understand each of the following input devices and why they are required Keyboard Microphone Optical mouse Touch screen (resistive, capacitive and infra-red)

Content

Starter PowerPoint Guide: Topic 1 Manual input devices The starter gives students an opportunity to consider the devices that they use in school and at home. Ask them what input devices are used in these devices. Some answers will be simple and generic, others such as games controllers will commonly be used. Whilst many of the examples (such as games controllers) are not on the specification, they are useful to elicit from students as they show an understanding of what input controllers are. Main Computer systems Make students aware of the overall structure of computer systems. Computer systems will always require a primary storage (typically RAM for working memory). They are also likely to need secondary non-volatile storage. Keyboards and keypads Keyboards and keypads are probably the most common form of data entry into a computer. They have the advantage of being easy to use and also require very little training. Specialist keyboards such as numeric keypads, ergonomic keyboards or gaming keyboards, have also been produced to suit various applications. The main drawbacks are that it is very easy to make typing errors and data entry is also very time consuming even if the user is able to touch-type. Keyboards are connected to the computer either by using a USB port or a wireless dongle. Tablets and mobile phones usually use some form of touchscreen virtual keyboard which can be even slower or more error-prone when entering data. How do keyboards work? It is important to note that the specification does not require that students understand the inner working of devices. In some cases we have kept these explanations in the topics as they are likely to be of interest to students. They will also help students understand whether devices will be suitable to a given context, which is a requirement of the specification. It would be useful here to have an old keyboard so that the students can see the inner workings of the device. Inspection of underneath the keys on a typical keyboard shows small holes in a plastic box; the keys have a small plastic peg which is the same shape and size as the hole. The keyboard consists of three layers of plastic: two layers are covered with electrical tracks and the middle layer is an insulating layer containing holes through which the pegs on the keys pass. When a key is pressed, the peg pushes through the hole and makes contact with a membrane containing electrical tracks. This completes a circuit and a signal is sent to the computer. The computer interprets this signal and works out which key was pressed. Pieces of rubber under a key make it bounce back to its original position so it can be pressed again. When the key bounces back, the electrical connection is also broken. Optical mouse A mouse can be mechanical or optical. Only optical mice are mentioned on the specification. An optical mouse shines a red LED onto a surface with the reflection being detected by a sensor to detect movement. A sensor takes a low-resolution image of the surface. As the mouse moves, images are continuously taken and compared with the previous image to calculate the direction of movement. Microphones A microphone consists of a diaphragm, a coil and a permanent magnet. When sound reaches the diaphragm, it vibrates backwards and forwards. This causes a coil to move backwards and forwards at the same time. As the coil moves, it changes the magnetic field produced by the permanent magnet. This change creates an electric current (in analogue form) which needs to be converted into a digital signal so the computer can understand the data being sent to it by the microphone. The ADC carries out this conversion. Once converted into digital, the computer can store or process this data. Hand out Worksheet 1 and ask students to complete Task 1. Topic 1 Worksheet 1 Topic 1 Worksheet 1 Answers Touch screens on tablets and mobile phones. There are four common technologies currently used by tablet and mobile phone manufacturers (the technologies are also used in a number of other applications): - Resistive - Capacitive - Infra-red This topic explains how each of the technologies work as they are listed separately on the specification, suggesting that students would need to understand the technological differences in order to choose the correct screen for a given context. Resistive Touchscreens using resistive technology are made of an upper layer of conducting polymer (usually polyester coated in indium tin oxide – ITO). There is also a bottom layer which made from glass with a conductive coating (using ITO). There is an insulating layer sandwiched between these two conductive layers. When the screen is pushed (by a finger or stylus), the two conducting layers make contact and complete an electric circuit. The position where the screen is touched is calculated by a microprocessor which interprets the signals sent out when the screen is touched. Strengths: - relatively inexpensive technology - can be used with bare finger, gloved hand or stylus Weaknesses: - not so good in strong sunlight; image sometimes poor relatively easy to scratch the polymer screen - can only support single touch (doesn’t support multi-touch functions) Capacitive All modern smartphones use capacitive touchscreens. Touchscreens using capacitive technology are made up of multiple layers of thin glass. The bottom of the first layer and top of the last layer of the glass are coated in ITO to make the surface conductive. There is an insulating layer of glass between these two conductive layers – this makes the screen act like a capacitor. When a finger touches the screen, the electrical field changes; the coordinates where the screen was touched is worked out by a microprocessor. Unlike resistive, capacitive technology allows multiple touches (enabling ‘pinching and zooming’ on a mobile phone and so on). Strengths: - supports multiple touch facility - has good contrast in strong sunlight - very durable technology Weaknesses: - can only be used with a bare finger or with capacitive pen - relatively expensive technology Infra-red Touchscreens using infra-red technology use a grid pattern of LEDs and photocells (light sensors). The LEDs shine IR light in front of the screen in a grid pattern. When the screen is touched the IR beams are interrupted – this indicates the coordinates where the screen was touched. A microprocessor calculates these coordinates and sends the information to the device. These are commonly used in fast food restaurant tills as the screen can then be coated in grease and scratch resistant rubber. Strengths: - good contrast in strong sunlight - very durable technology - can be used with bare finger, gloved hand or stylus Weaknesses: - ambient light can sometimes cause problems if screen is scratched - only supports single touch (doesn’t support multi-functions) Ask students to complete Task 2 on Worksheet 1. Plenary Ask students to discuss the answers to the questions on the slide in pairs. Once they have had time to consider their answers, ask pairs to feed back their answers. Hand out Homework 1. Topic 1 Homework 1 Topic 1 Homework 1 Answers

Topic 2 Scanners and cameras

Objectives: Understand each of the following input devices and why they are required Barcode scanner QR code scanner Digital camera Two-dimensional (2D) and three-dimensional (3D) scanners

Content

Starter PowerPoint Guide: Topic 2 Scanners and cameras The starter asks students to review the overall diagram of a computer system which they encountered in the last topic. Once students have considered each of the boxes in the diagram, check their answers with those given on the following slide. Main Barcode readers Barcodes are one of the most common methods of representing data and have been around for many years. An excellent website used to create barcodes is available at: www.barcode-generator.org. The specification has been updated, and it is unlikely that students will be expected to understand exactly how barcodes work. However, the topic is interesting and aids the understanding of the technology. The important point for students to have an appreciation of, is why barcodes readers are required and the likely contexts in which they are used today. Why use barcodes? Take students through the different contexts where barcodes are commonly used. Ask students if they have any further examples where they have seen barcodes used. For example, they may have noticed barcodes (or QR codes) on internal parts of a computer as they are used for identification purposes. QR (Quick Response) codes QR codes can be used to provide information or to generate a web link which takes the user directly to a website once the QR code is scanned. They can also create: - VCards to automatically insert contact information into a smart phone - Emails that will send a predefined email to a registered address. The example codes on the slide have the following content: - “QR codes store up to 4296 characters.” - “QR codes store website addresses, email addresses or text.” Ask students where they have seen QR codes used and take them through the example uses given on the slides. Hand out Worksheet 2 and ask students to complete Task 1 and optionally Task 3. Topic 2 Worksheet 2 The worksheet contains an optional task 3. This is not required for the new specification, but many teachers have said that students have enjoyed the task, so we have kept it in as an optional activity. If you wish to undertake this task, then carry it out in addition to Task 1. 2D scanners Take students through how 2D scanners work. They are used to scan photographs and also embedded into both home and business photocopiers. Due to the ubiquity of digital cameras, 2D scanners are not used to the same degree that they once were. 3D scanners 3D scanners scan solid objects in the x-y-z planes producing images in a computer-readable form representing the scanned object. They are used in CAD, for example and are the basis behind 3D printer technology covered in the printers topic. Tomography Some 3D scanners just scan the surface of an object whilst others are able to scan inside the object itself. These scanners make use of a process known as tomography which ‘slices’ up a solid object to produce hundreds of very thin slices (often less than 0.1 mm thick). Examples include CT and MRI scanners. Objects such as aircraft parts can also be 3D scanned to see if there are imperfections, such as developing cracks, inside the material. Play the video “How to use 3D scanning for dent detection” [2m24s] https://www.youtube.com/watch?v=TYB1YZIGwLU Digital cameras Digital cameras make use of a special sensor known as a CCD (Charge-coupled device). These sensors often contain millions of light sensors to capture the millions of pixels required in an image. Show students the example of how an image is divided up into pixels. Explain that a high-resolution camera would need to detect more pixels on the CCD. Ask students to complete Task 2 on Worksheet 2. Plenary Ask students to answer the questions in pairs. The final question asks them to consider the technology that has most revolutionised our lives. There is no right or wrong answer to this question. They may choose to pick a technology such as MRI which has saved lives. Equally, digital cameras have transformed photograph whilst the humble barcode has made huge changes in the efficiency of shops and supermarkets, helping to reduce waste and keep costs low. Hand out Homework 2. Topic 2 Homework 2 Topic 2 Homework 2 Answers

Topic 3 Sensors

Objectives: Understand the meaning of a sensor Understand the purpose of the following sensors Acoustic Accelerometer Flow Gas Humidity Infra-red Level Light Magnetic field Moisture pH Pressure Proximity Temperature

Content

Starter PowerPoint Guide: Topic 3 Sensors Before teaching individual sensors, it is worth students considering the physical properties that exist in the world. If they don’t understand the question, give one or two other examples such as pressure and light intensity. Once you have gone through the answers, explain that many of the sensors simply use the name of the physical property they are trying to detect. Main Sensors Take the students through the definition of a sensor. Explain to them that sensors will almost always create data in analogue form. Computers though, operate with binary and limited memory, so require the data in a digital form. Hence an Analogue to Digital Convertor (ADC) is required. Car temperature sensors Take students through the example of a car temperature sensor. The question on the slide asks students to consider other sensors that are used in cars. Tyre pressure sensors and proximity sensors are given as examples as these are likely to be present in the cars that students travel in. Students may also like to consider other sensors such as the pressure sensor used to detect if someone is sitting on a seat and doesn’t have a seat belt on. Many other sensors are used within the engine with further sensors, such as LIDAR, being required for cars that are self-driving. Accelerometers Accelerometers are mainly used to determine if a device is in portrait or landscape mode. As they are able to detect movement, they are used in some games, such as driving games as a control method. They are also used on iOS devices, where shaking the device can be used to undo an operation. Other sensors Many sensors are given on the specification. You may wish to cover one side of the slide and allow students to suggest applications for each type of suggestion before giving the suggestions on the slide. Hand out Worksheet 3 and ask students to complete Task 1. Topic 3 Worksheet 3 Topic 3 Worksheet 3 Answers Security system Ask students to consider the three sensors that are part of an art gallery’s security system and give an explanation of what each sensor does, along with it’s advantages and disadvantages. Answers are given on the following slide. Sensor feedback and how it works Take students through these slides that cover how these sensors work as part of a larger system. This allows students to see how input and output devices are all used together to make a meaningful system. Water treatment Students are less likely to have appreciated how a pH sensor is required as these are mainly used within industry. Play the video “Flow-Tech: Introduction to pH control loops” [3m15s] https://www.youtube.com/watch?v=Z-SlGIiIM4U This video allows students to see how control loops that use pH sensors are used within water treatment. It also gives a cross-curricular opportunity with the chemistry they are likely to be studying at this level. Ask students to complete Task 2 and Task 3 on Worksheet 3. Plenary Ask students to go through the slide and describe how an automated outdoor light turns on. Two sensors are required to solve this problem. Take students through the following slide that gives the answers. Hand out Homework 3. Topic 3 Homework 3 Topic 3 Homework 3 Answers

Topic 4 Output devices

Objectives: Understand what is meant by an output device Understand each of the following output devices and why they are required Speakers Light emitting diode (LED) screens Liquid crystal display (LCD) projectors Liquid crystal display (LCD) screens Digital light processing (DLP) projectors Actuators

Content

Starter PowerPoint Guide: Topic 4 Output devices The starter is similar to the one which was given in the first topic on input devices. This time, ask students to consider the devices that allow the computer to output information. If they find this task easy, you may wish to ask them to try to create a definition of the term “output device”. Answers are given on the following slide. There are many examples than this, such as fans and heaters. These are the only output devices that students need to know on the specification. Printers are covered in the next topic. Main Speakers and headphones Sound from a computer can be heard using the following group of devices: DAC (digital to analogue converter) amplifier (increases the signal) loudspeaker (produces the sound) Refer to the diagram on the slides which shows how the above three devices connect. Actuators Actuators are very commonly used to make mechanisms and systems move. They are not the same as motors. A motor will simply spin repeatedly when turned on. For instance, a fan is an example of a motor spinning. A stepper motor by contrast is an example of an actuator. It rotates a specific number of degrees. This makes it suitable as an actuator to open or close a valve for example. Watch the video “What is an actuator?”. [5m09s] https://www.youtube.com/watch?v=LHn7O6PUaoY Ask students to suggest other situations that use actuators and take them through the Answers slide that follows. Monitors/screens Screens are mostly LCD (liquid crystal display/diode) technology. Tiny diodes are grouped together in threes to make up each pixel (picture elements). The group is made up of red, green and blue diodes. Because LCD doesn’t produce any light, screens using this technology need some form of back-lighting. Earlier LCD screens used CCFL (Cold Cathode Fluorescent Lamps) to supply the light. More modern systems use a matrix of tiny LEDs (light emitting diodes) to back-light the LCD screen. This is the technology used in most mobile phones and tablets if they use LCD screens. Using LEDs to back-light LCD screens has a number of advantages over the older CCFL technology: they reach their maximum brightness almost immediately they give out a very white light which makes: the image sharper the colours more vivid for more realistic colours (CCFL gave a yellowish tint) they produce a brighter light which leads to better picture definition since LEDs are very small, this allows the screens to be much thinner in construction they last almost indefinitely which makes the screens much more reliable they consume very little power and therefore produce very little heat as well and reduce running costs Hand out Worksheet 4 and ask students to complete Task 1. Topic 4 Worksheet 4 Topic 4 Worksheet 4 Answers - Projectors - Digital projectors are used to show computer output on larger screens for larger audiences or display purposes. When they are used to display presentations, the output often includes sound (e.g. music or voice-overs), animation (e.g. slide transitions) or video – they also sometimes use internet links which when clicked on display the website embedded in the presentation). - There are two common forms of projector: Digital light projectors (DLP) Liquid Crystal Display (LCD) projectors DLP projectors Although the inner workings of devices are not specifically requested on the specification, students are expected to know why each of the devices are required. As such, an understanding of the differences in the DLP and LCD technologies will be helpful in this. Go through the slides that describe how DLP and LCD projectors work. There is a lot of cross-over in the advantages and disadvantages of each technology as each improve with time. As such, a suitable activity is to get students to research the advantages and disadvantages of the current ‘best in class’ DLP and LCD projectors. Ask students to complete Task 2 on Worksheet 4. Plenary Ask students to answer the questions given on the plenary slide which review the key concepts given in the lesson. Once each pair has had a chance to discuss the answers, ask them to feedback to the class. Suggested answers are given on the following slide. Hand out Homework 4. Topic 4 Homework 4 Topic 4 Homework 4 Answers

Topic 5 Printers

Objectives: Understand each of the following printing devices and why they are required Inkjet printer Laser printer 3D printer

Content

Starter PowerPoint Guide: Topic 5 Printers The starter slide shows a large inkjet printer. Ask students to name different technologies that are used in printers. It’s likely that they will have seen or had experience with inkjet, laser and 3D printers. They are also likely to have used thermal printers when getting till receipts. Other printers exist. For example, lithographic printers will create a plastic or metal plate (which is an expensive process), but from that plate, they will be able to create multiple high-quality copies very cheaply. This technique is used for books, magazines and brochures. Main - Inkjet printers - Demonstrate the internal parts of an old inkjet printer if you have one. Tell students that they contain cartridges of liquid ink, usually in four colours: cyan, magenta, yellow and black (CMYK). - The print head assembly contains nozzles which spray ink droplets onto the paper and a belt is used to move the print head across the paper. - Stepper motors are used to move the print head across the page and to feed paper into the printer. - How are the ink droplets produced? - There are two common methods of producing ink droplets to be sprayed on the paper using ink jet printers. The two methods are: Thermal bubble Piezoelectric crystal - The operation of both systems is quite different: - Thermal bubble - This produces droplets of ink using the following method: - behind the print nozzles there are tiny resistors that create heat (the temperature momentarily heats to over 300°C) - the heat causes the ink to instantly vaporise and form a tiny bubble - as this bubble expands, ink is ejected onto the paper - as it cools, the bubble collapses and draws in fresh ink - this process continues until the page is printed - a typical print head has over 300 tiny nozzles - Piezoelectric crystal - This produces droplets of ink using the following method: - a small piezoelectric crystal is located at the rear of each ink reservoir of each nozzle - when the crystal receives a small electric current, it begins to vibrate in and out - as it vibrates inwards, ink is ejected onto the paper - as it vibrates outwards, ink is drawn in again to replace the ink sprayed - Highlight the benefits and drawbacks of the two technologies. Hand out Worksheet 5 and ask students to complete Task 1. Topic 5 Worksheet 5 Topic 5 Worksheet 5 Answers - Laser printers - Laser printers use dry toner rather than liquid ink as in inkjet printers. They rely heavily on the use of electrostatic charges to produce text and images on paper. Unlike inkjet printers, they produce a whole page at a time rather than print line by line. Printing with laser printers is usually faster as a print-head is not required to move backwards and forwards across the page. Instead, a drum simply rotates as the page moves through. Laser printers can print in colour by using four colours of toner (CMYK). Ask students to complete Task 2 on Worksheet 5. 3D printers 3D printers are increasingly finding all kinds of uses. If students have one in school, ask them about their experiences and how it works. Show the videos: 3D printing – What is it? [1m15s] https://www.youtube.com/watch?v=EF8CNR-gcXo Hershey’s 3D printer prints chocolate [1m42s] https://www.youtube.com/watch?v=slksoDJPJ94 Take students through the slides to show how the printers work and how they can be used. In particular, highlight how useful 3D scanners can be in creating the 3D models that are used by 3D printers. Applications in medicine After going through the slide, play the video: 3D printed prosthetic hands [3m03s] https://www.youtube.com/watch?v=Cl8ijPGEKO8 Ask students to complete Task 2 on Worksheet 5.

Plenary The plenary gives students a chance to consider the different types of printer available, how they work and examples of where they are required. Remind them that the exam board may well give them a context and then expect them to choose the correct printer (or input/output devices). Go through some suggested answers on the following slide. Hand out Homework 5. Topic 5 Homework 5 Topic 5 Homework 5 Answers

Final assessment

Learning Outcomes: - Students will apply their knowledge in answers to a range of questions be able to identify areas of strength and any gaps in their understanding of impacts of digital technology on wider society

Content

Students should complete the Assessment Test. Unit 4 Assessment Test.docx Unit 4 Assessment Test Answers.docx The test has been designed to be printed and answered by hand.

Acknowledgements

The authors and publisher would like to thank all contributors for their kind permission to reproduce their photographs or images, screenshots of their websites or other copyright material in the PowerPoint Guides.

Photographic images copyright © Shutterstock

Image of TASCAM sound recorder © Birgit Reitz-Hofmann / Shutterstock.com

Image of NEC Birmingham © Ceri Breeze / Shutterstock.com

Every effort has been made to trace the copyright holders and we apologise in advance for any unintentional omissions. We would be pleased to insert the appropriate acknowledgement in any subsequent edition of this publication.

This material contains links to relevant websites featured in the teacher’s guide. Every effort has been made to ensure that at the time of distribution, the links remain unbroken, the material remains up-to-date and that links are not inadvertently linked to sites that could be considered offensive. PG Online cannot be held responsible for the content of any website mentioned in this material. It is sometimes possible to find relocated sites by typing the original URL into a browser. Any errors should be reported directly to support@pgonline.co.uk and changes will be made in any subsequent editions of the material.

Artwork

Atmospheres – Tidal Island

© 2018 Neil Canning

Mixed media / paper 21 × 29.5 cm

www.neilcanning.com